Experiment 2 : 		Develop a program to deploy intrusion detection system using ultrasonic and sound sensors

Date : 19.08.2025

Aim:

To develop the intrusion detection system using ultrasonic and sound sensors

Components Required:

| S.No | Components Name | Range/Rating | Quantity |
| --- | --- | --- | --- |
| 1 | Universal Bread Board |  | 1 |
| 2 | Arduino Uno board |  | 1 |
| 3 | HC-SR04 Ultrasonic Sensor |  | 1 |
| 4 | Sound Sensor (Microphone Module) |  | 1 |
| 5 | Buzzer or LED |  |  |
| 6 | 12V Adaptor |  | 1 |
| 7 | Power jack |  | 1 |
| 8 | USB Cable |  | 1 |
| 9 | Jumper Wires |  | required |

HC-SR04 Ultrasonic Sensor: Emits ultrasonic waves and measures the time taken for the echo to return, allowing distance calculation to detect objects or intruders within a defined range.

Sound Sensor (Microphone Module): Detects sound levels and can be used to identify unusual noises associated with an intrusion.

Buzzer or LED: Provides an audible or visual alarm when an intrusion is detected.

Hardware Procedure:

- Strategically place the ultrasonic sensor to cover the desired detection area and the sound sensor to monitor for unusual noises.

- Power the Arduino using a USB cable or an external power supply

Hardware Setup:

Setting up the hardware

- Ultrasonic sensor connection:

  - Connect the VCC pin to a 5V pin on the Raspberry Pi.

  - Connect the GND pin to a ground pin on the Raspberry Pi.

  - Connect the TRIG pin to a GPIO pin on the Raspberry Pi (e.g., GPIO 20).

  - Connect the ECHO pin to another GPIO pin on the Raspberry Pi (e.g., GPIO 26) through a voltage divider (using resistors to lower the 5V output of the sensor to the 3.3V acceptable by the Raspberry Pi's GPIO pins).

- Sound sensor connection:

  - Connect the power and ground pins to the Raspberry Pi's corresponding pins.

  - Connect the data/output pin to an available GPIO pin on the Raspberry Pi (e.g., GPIO 17).

- Output devices (buzzer/LEDs):

  - Connect the buzzer's positive terminal to a GPIO pin and the negative terminal to ground.

  - Connect the anode (longer leg) of an LED to a GPIO pin through a current-limiting resistor, and the cathode (shorter leg) to a ground pin

Connection Diagram:-

Program

// C++ code

//

int input = 0;

long readUltrasonicDistance(int triggerPin, int echoPin)

{

pinMode(triggerPin, OUTPUT);  // Clear the trigger

digitalWrite(triggerPin, LOW);

delayMicroseconds(2);

// Sets the trigger pin to HIGH state for 10 microseconds

digitalWrite(triggerPin, HIGH);

delayMicroseconds(10);

digitalWrite(triggerPin, LOW);

pinMode(echoPin, INPUT);

// Reads the echo pin, and returns the sound wave travel time in microseconds

return pulseIn(echoPin, HIGH);

}

void setup()

{

Serial.begin(9600);

pinMode(8, OUTPUT);

pinMode(9, OUTPUT);

pinMode(10, OUTPUT);

}

void loop()

{

input = 0.01723 * readUltrasonicDistance(6, 5);

Serial.println(input);

if (input < 100) {

digitalWrite(8, HIGH);

digitalWrite(9, HIGH);

digitalWrite(10, LOW);

} else {

digitalWrite(8, LOW);

digitalWrite(9, LOW);

digitalWrite(10, HIGH);

}

delay(10); // Delay a little bit to improve simulation performance

}

Working Observation:

- Create a loop to read both sensor values continuously. Define a distance threshold for the ultrasonic sensor. Use conditional logic to trigger an alarm if an object is within the threshold and sound is detected.

- Test the system by simulating an intrusion and adjust the ultrasonicThreshold and soundThreshold values in the code as needed to optimize detection sensitivity and minimize false alarms.

- Activate a buzzer or LEDs upon intrusion detection.

RESULT: intrusion detection system is successfully implemented with Arduino microcontroller Board.