Experiment 5: 		Develop a program to deploy smart street light system using LDR with Arduino Board

Date : 02.09.2025

Aim:

To develop the program to deploy smart street light system using LDR sensorwith Arduino Board.

Components Required:

| S.No | Components Name | Range/Rating | Quantity |
| --- | --- | --- | --- |
| 1 | Universal Bread Board |  | 1 |
| 2 | Arduino Uno board |  | 1 |
| 3 | LDR sensor or photoresister |  | 1 |
| 4 | Resister | 10 kohm | 1 |
| 6 | Resister | 330 ohm | 1 |
| 7 | LED |  | 1 |
| 8 | USB Cable |  | 1 |
| 9 | Jumper Wires |  | required |

Hardware Setup:

Connect LED with pin D10 and GND

Connect LDR pin with A0 and other pin with +5vcc

Arrange the wires as in the following diagram

Connection Diagram: -

Program

// C++ code

//

int LDR_var = 0;

void setup()

{

pinMode(A0, INPUT);

Serial.begin(9600);

pinMode(10, OUTPUT);

}

void loop()

{

LDR_var = analogRead(A0);

Serial.println(LDR_var);

if (LDR_var < 500) {

digitalWrite(10, HIGH);

} else {

digitalWrite(10, LOW);

}

delay(10); // Delay a little bit to improve simulation performance

}

Working Observation:

- Initially the LED is on glowing mode, since the light intensity is less than the value 500.

- It is observed that when the light intensity increases then the LDR will increases it’s value simultaneously that can be viewed through the serial monitor.

- So, as the light intensity increases the LDR value also increases, therefore the  LED which is connected in the pin D10 will stop glowing.

RESULT: smart street light system is deployed using LDR sensor successfully and executed with Arduino microcontroller Board.