Experiment 6: 		Develop a program to classify dry and wet waste using a moisture sensor (DHT22) with Arduino Board

Date : 09.09.2025

Aim:

To develop the program to classify dry and wet waste using a moisture sensor with Arduino Board.

Components Required:

| S.No | Components Name | Range/Rating | Quantity |
| --- | --- | --- | --- |
| 1 | Universal Bread Board |  | 1 |
| 2 | Arduino Uno board |  | 1 |
| 3 | Moisture sensor | DHT22 | 1 |
| 4 | Resister | 330 ohm | 1 |
| 5 | LED |  | 1 |
| 6 | USB Cable |  | 1 |
| 7 | Jumper Wires |  | required |

Hardware Setup:

Connect LED with pin D10 and GND

Connect DHT22 pin with A0 and other pin with +5vcc and GND

Arrange the wires as in the following diagram

Connection Diagram: -

Program

// C++ code

//

int m_data = 0;

void setup()

{

pinMode(A0, INPUT);

Serial.begin(9600);

pinMode(10, OUTPUT);

}

void loop()

{

m_data = analogRead(A0);

Serial.println(m_data);

if (m_data < 21) {

digitalWrite(10, HIGH);

} else {

digitalWrite(10, LOW);

}

delay(10); // Delay a little bit to improve simulation performance

}

Working Observation:

- Initially the LED is glowing mode, since the moisture sensor value is less than the specified value I,e 21.

- It is observed that when the moisture sensor value increases then the sensor value will increases that can be viewed through the serial monitor.

- So, the dryness increases the soil moisture value also increases, therefore the  LED which is connected in the pin D10 will stop glowing.

RESULT: The dry and wet is classified by employing the soil moisture sensor with Arduino microcontroller Board and executed successfully.